- ACTIVITY IDEA: How safe is my house?

- What you will need:

- Wire coils/springs x 4

- 10 cm x 10 cm x 2 cm wooden block x 2

- House blocks 3 cm x 3 cm x 3 cm x 4

- Isolators of different textures and properties (jelly/jubes, marshmallows, rubber stoppers, polystyrene, water in zip lock bag, ball bearings or marbles in zip lock bag)

- Blu-Tack

Experimental conditions:

1 – no isolators (control)

2 – hard isolators

3 – flexible isolators

Hypothesis:

Predict how long it will take for the house to fall off the base for the different experimental conditions.

Instructions:

- Build a model house on a wooden block (or paper plate the same size – plastic is too slippery). The house should consist of outer walls only. Use 3 cm3 blocks or empty matchboxes. The wooden block represents the floor of the house.

View from top

- Make four wire coils or springs (coils need to be large – wind the wire around a whiteboard marker about 16 times). Attach the coils with Blu-Tack to the underside of the second wooden block or paper plate. If using a paper plate, cut it to size. This wooden block represents the ground under the house.

- Select a suitable material for base isolators e.g., marshmallows, jubes, rubbers, wooden blocks, polystyrene, water in plastic bag, ball bearings in plastic bag.

- Place at least four base isolators on the centre of the wooden block representing the ground.

- Mount the house onto the base isolators. You will need to use Blu-Tack to give some grip.

Blocks for house

Base of house

Base isolators

springs

Ground

Side view

- Push the ground block backwards and forwards gently to simulate earthquake motion. The force used must be consistent each time this is repeated.

- Time in seconds how long it takes for the house blocks to fall off the floor of the house. Record this time in a table (see Results).

- Repeat steps 4 to 7 but use different materials as base isolators each time, or omit them altogether (control condition).

Results:

| BASE ISOLATOR MATERIAL | PROPERTIES OF MATERIAL | PREDICTED TIME FOR HOUSE TO FALL (s) | ACTUAL TIME FOR HOUSE TO FALL (s) |
| --- | --- | --- | --- |
| Hard jubes |  |  | Trial 1: Trial 2: Trial 3: Average: |
| marshmallows |  |  | Trial 1: Trial 2: Trial 3: Average: |
| rubber |  |  | Trial 1: Trial 2: Trial 3: Average: |
| Polystyrene |  |  | Trial 1: Trial 2: Trial 3: Average: |
| Water |  |  | Trial 1: Trial 2: Trial 3: Average: |
| Ball bearings |  |  | Trial 1: Trial 2: Trial 3: Average: |

Conclusion:

Which material provided most protection against earthquakes?

Discussion:

- What property gave the best protection against earthquake damage?

- How could you improve this investigation?

- What would happen if your house was two or three storeys high?

- What effect would base isolators have if the ground was loose or sandy, rather than solid?